SWS Internet of Things Access Interface Protocol (Open)

# summary

## 1.1 Writing purpose

This document describes the interaction process between battery and IoT platform (including business server) and the interface protocol.

This article applies to system analysis designers, solution testers, product developers, and product testers.

## 1.2 Technical scheme

The device and the background connect and communicate through MQTT, and the device logs in the MQTT server with the configured MQTT address, account number and password. After the MQTT connection is established, normal data reporting, data query, device control operation and so on are conducted.

The message content is sent in the payload of MQTT, and the data format of payload is defined, see later article section.

# Interface definition

The data format of this Agreement is defined as follows:

| classify | length | explain |
| --- | --- | --- |
| Message Header | 6 Bytes | headers |
| Message Body | N Bytes | The message body |

The message header is defined as follows

| Field name | option | form | length | explain |
| --- | --- | --- | --- | --- |
| Start Code | M | V | 2 Byte | The start character is fixed at 0x4350. |
| Protocol Version | M | V | 1 Byte | Protocol version, fixed value 0x11 |
| Sequence Number | M | V | 2 Byte | Message sequence number, unsigned integer, used to identify the sequence of messages, if the maximum value (65535) has been reached, 0 starts over |
| Transaction Id | M | V | 1 Byte | Transaction ID, an unsigned integer, is used to identify a transaction. It is filled in when the sender sends a request message, and the receiver's response message returns the Transaction Identity of the request message. The Transaction Identity of the resent message is the same as the Transaction Identity of the initial message. If the maximum value (255) has been reached, it will restart from 0. |

The message physique for each message is defined in each of the [2.X] sections described below.

## 2.1 Battery dynamic data reporting

### 2.1.1 Interface specification

| Topic | /${productName}/${deviceName}/user/batPropertyRprt |  |  |
| --- | --- | --- | --- |
| direction | Device→IoT Platform | Permissions | release |

### 2.1.2 Interface description

${productName}: Fixed as "SW _ GPS"

${deviceName}: GPS Coding

The format definition of the message is shown in [3.1  Battery Prop s】

2.2 Static battery data reporting

### 2.2.1 Interface specification

| Topic | /${productName}/${deviceName}/user/batPropertyExtRprt |  |  |
| --- | --- | --- | --- |
| direction | Device→IoT Platform |  |  |

### 2.2.2 Interface description

${productName}: Fixed as "SW _ GPS"

${deviceName}: GPS Coding

The format definition of the message is found in【3.2  Battery Prop s Ext】

## 2.3 Battery location data reporting

### 2.3.1 Interface specification

| Topic | /${productName}/${deviceName}/user/batPositonRprt |  |  |
| --- | --- | --- | --- |
| direction | Device→IoT Platform | Permissions | publish |

### 2.3.2 Interface description

${productName}: Fixed as "SW _ GPS"

${deviceName}: The GPS coding

Report the battery's recent location history, up to 10 records

The format definition of the message is shown in [4.6  Position Info List】

## 2.4 Query the battery dynamic data

### 2.4.1 Interface specification

- Device subscription (listening query instructions)

| Topic | /${productName}/${deviceName}/user/batPropertyReq | authority | subscribe to |
| --- | --- | --- | --- |
| direction | IoT platform → device side | pattern | asynchronous |

- Device-side subscription (returns the query results)

| Topic | /${productName}/${deviceName}/user/ batPropertyRsp | authority | publish |
| --- | --- | --- | --- |
| direction | IoT platform → device side | pattern | asynchronous |

### 2.4.2 Interface description

${productName}: Fixed as "SW _ GPS"

${deviceName}: The GPS coding

This interface is used to actively query the dynamic data of the device. The reported data structure is consistent with [Battery Dynamic Data Reporting].

Request message has no message body (send only message header)

The data format of the response message is shown in [3.1  Battery Prop s】

## 2.5 Query the battery static data

### 2.5.1 Interface specification

- Device subscription (listening query instructions)

| Topic | /${productName}/${deviceName}/user/batPropertyExtReq | authority | subscribe to |
| --- | --- | --- | --- |
| direction | IoT platform → device side | pattern | asynchronous |

- Device-side subscription (returns the query results)

| Topic | /${productName}/${deviceName}/user/ batPropertyExRsp | authority | publish |
| --- | --- | --- | --- |
| direction | IoT platform → device side | pattern | asynchronous |

### 2.5.2 Interface description

${productName}: Fixed as "SW _ GPS"

${deviceName}: The GPS code

This interface is to actively query the static data of the device, and the query data information is consistent with the format and content of the battery static data reporting information.

Request message has no message body (send only message header)

The data format of the response message is shown in:【3.2  Battery Prop s Ext】

## 2.10 Device Shutdown command

### 2.10.1 Interface specification

- Device-side subscription (listening for query instructions)

| Topic | /${productName}/${deviceName}/user/batteryPowerOffReq | authority | subscribe to |
| --- | --- | --- | --- |
| direction | IoT platform → device side | pattern | asynchronous |

### 2.10.2 Interface description

${productName}: Fixed as "SW _ GPS"

${deviceName}: The GPS code

Instructs the battery to perform shutdown processing.

Scenario 1: After receiving a shutdown request from the battery due to low power, the IoT platform sends a shutdown command to the battery

Scenario 2: In scenarios such as battery storage, operation and maintenance personnel remotely shut down or put the battery in a static state

See the message format【3.8 Battery PowerOff Command Param】

### 2.10.5, response data format

No response

## 2.11 Remote control of the equipment

### 2.11.1 Interface specification

- Device-side subscription (listening for query instructions)

| Topic | /${productName}/${deviceName}/user/bmsCtrReq | authority | subscribe to |
| --- | --- | --- | --- |
| direction | IoT platform → device side | pattern | asynchronous |

- Device-side subscription (return of processing results)

| Topic | /${productName}/${deviceName}/user/bmsCtrReq | authority | publish |
| --- | --- | --- | --- |
| direction | IoT platform → device side | pattern | asynchronous |

${productName}: Fixed as "SW _ GPS"

${deviceName}: The GPS coding

Request/response message data format see in【3.7 Battery Remote Control Command P aram/Result】

# 3 Data protocols

## 3.1  Battery Props

Contains battery serial number (VIN code), battery status, alarm status, 4G module status, GPS location data that can be obtained by battery BMS vendors

| Field name | length | explain |
| --- | --- | --- |
| Product SN | LV | See 4.1 |
| Battery Status | 4 Bytes | Start and stop state, electrical status and alarm state, see 4.5 |
| Battery Data | L V | Data for all battery attributes, see 4.4 |
| 4G·Net Info | LV | For the working condition data of the GPS module, see the 4.10 |

## 3.2  Battery Props Ext

Inactive battery data

| Field name | length | explain |
| --- | --- | --- |
| Product SN | LV | Battery factory number (battery serial number?), see 4.1 |
| GPS SN | LV | GPS factory number, data format please refer to 【4.12】 |
| GPS IMSI | LV | GPS · IMSI, for the data format, please refer to 【4.12】 |
| GPS IMEI | LV | GPS · IMEI, for the data format, please refer to 【4.12】 |
| GPS Software Ver | LV | GPS · Software version number, for data format, please refer to【4.12】 |
| GPS Hardware Ver | LV | GPS · Hardware version number, for data format, please refer to 【4.12】 |
| BMS SN | LV | BMS · Factory number, for data format, please refer to 【4.12】 |
| BMS Software Ver | LV | BMS · Software version number, for data format, please refer to 【4.12】 |
| BMS Hardware Ver | LV | BMS · Hardware version number, for data format, please refer to 【4.12】 |

## 3.3 IoT-Config

| Field name | length | explain |
| --- | --- | --- |
| Prop Report Period | 2 Bytes | Active data & GPS location reporting period (interval), in seconds, 0 means turning off the periodic reporting function |
| Prop Report Distance | 2 Bytes | Active data & GPS location·Movement reporting threshold, in meters. 0 means turning off the movement reporting function |
| Prop- Ext Report Time | 3 Bytes | Non-static data·Daily scheduled reporting time, see 4.11, the value of 250000 means turning off the regular reporting function |
| Gps Required on Move | 1 Byte | The minimum number of positioning stars required when the battery is moving |
| Gps Required on Static | 1 Byte | The minimum number of satellites required for positioning when the battery is stationary |

## 3.4  Update IoT-Config Param/ Result

Not supported yet

## 3.5  Update BMS Config Param/ Result

Not supported yet

## 3.6  Update BMS Info Command Param/ Result

Not supported yet

## 3.7 Battery Remote Control Command Param/Result

| Field name | length | explain |
| --- | --- | --- |
| Control Type | 1 Byte | 1: discharge switch operation, 2: charging switch operation, 3: battery static mode setting |
| V alue | 1 Byte | The definition of Value is shown in the following table |

| Control Type | Value |  | explain |
| --- | --- | --- | --- |
| 1 | 0 | Allow discharge |  |
|  | 1 | No discharge |  |
| 2 | 0 | Allow charging |  |
|  | 1 | No charging |  |
| 3 | 0 | Standing mode is not allowed |  |
|  | 1 | Allow static mode |  |

## 3.8  Battery PowerOff Command Param/Result

Remote shutdown (power-off) command, scenario example: After the IoT platform receives a shutdown request from the battery due to low power, it sends a shutdown command to the battery.

| Field name | length | explain |
| --- | --- | --- |
| Power Off F l ag | 1 Byte | The battery side performs the shutdown commands only when the value is 0x03 |
| Cause | 1 Byte | Shutdown reason code: 1-Low power 2-battery in standing mode |

## 3.9  Excute Battery OTA Param

reservation

## 3.10  Excute Battery OTA Result

reservation

## 3.11  Update Connect-Config Param/ Result

## reservation

## 3.12  Power OffApply Param

# reservation

# 4 Metadata structure

## 4.1  Product  Serial  Number

The product serial number (VIN code in the GPS-BMS protocol) is the factory code burned into the BMS board (see Figure 4-1-1).

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | explain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of Product Serial N umber |  |  |  |  |  |  |  | octet 1, value is the number of bytes, 0 value means BMS has not set the battery product serial number |
| Product Serial N umber |  |  |  |  |  |  |  | octet 2 ~ n, ASCII encoding |

## 4.3  Product  Type

Currently only includes battery strings

| Field name | length | explain |
| --- | --- | --- |
| Cell Quantity | 1 Byte | Value type, unit: a, indicating the number of cells of the cell |

## 4.4  Battery Data

The status of each battery attribute (dynamically changing data).

| Field name | length | explain |
| --- | --- | --- |
| Current | 2 Bytes | Present current. Network byte order. Signed number, positive for charging and negative for discharging, unit is 0.1A. 0xFE85 The discharge current is 37.9A. |
| Cell Voltage List | LV | List of the cell voltage, see 4.7 |
| BMS Temperature List | LV | List of temperature values for the BMS detection line, see 4.9 |
| Cell Temperature List | LV | List of the temperature values of the cell detection line, see 4.9 |
| Loop Times | 2 Bytes | Number of charge and discharge cycles, an unsigned integer. |
| Remaining Capacity | 2 Bytes | Remaining power, in 0.1Ah, unsigned integer. |
| Total Capacity | 2 Bytes | Present capacity, power, unit: 0.1Ah. , unsigned integer. QA: What is the design capacity when it leaves the factory (denominator of SOH)? What is the maximum capacity that can be charged at present (denominator of SOC?) |

## 4.5  Battery  Status

The battery's various functions are turned on/off, execution/standby, and various warning and abnormal conditions.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | explain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rsv | Rsv | Rsv | Rsv | Rsv | Rsv | Rsv | Rsv | octet 1 |
| Bit_23 | Bit_22 | Bit_21 | Bit_20 | Bit_19 | Bit_18 | Bit_17 | Bit_16 | octet 2 |
| Bit_15 | Bit_14 | Bit_13 | Bit_12 | Bit_11 | Bit_10 | Bit_9 | Bit_8 | octet 3 |
| Bit_7 | Bit_6 | Bit_5 | Bit_4 | Bit_3 | Bit_2 | Bit_1 | Bit_0 | octet 4 |

Bit 24 to Bit 31: reserved.

| Field name | length | explain |
| --- | --- | --- |
| Bit_0 | 1 bit | 【Charging status】 0: Idle 1: Charging |
| Bit_1 | 1 bit | 【Full state】 0: Not filled 1: already filled |
| Bit_2 | 1 bit | 【Charge overcurrent】 0: No (or removed), 1: occurred and not lifted |
| Bit_3 | 1 bit | 【Cell overvoltage】 0: No occurred (or removed) corresponding event, 1: occurred and not removed |
| Bit_4 | 1 bit | 【Discharge state】 0: Idle 1: is discharging |
| Bit_5 | 1 bit | 【Alarm · short circuit alarm】 0: the corresponding event has not occurred (or removed), 1: occurred and not lifted |
| Bit_6 | 1 bit | 【Alarm · discharge overflow]】0: No (or removed) corresponding event, 1: occurred and not lifted |
| Bit_7 | 1 bit | 【Alarm · cell undervoltage】 0: not occurred (or lifted) corresponding event, 1: occurred and not lifted |
| Bit_8 | 1 bit | 【Alarm · cell detection line open circuit】, 0: no occurred (or removed) corresponding event, 1: occurred and not removed |
| Bit_9 | 1 bit | 【Warning · temperature detection line open】, 0: not occurred (or removed) corresponding event, 1: occurred and not removed |
| Bit_10 | 1 bit | 【Alarm · cell high temperature】 Cell temperature is too high, 0: no occurred (or removed) corresponding event, 1: occurred and not removed |
| Bit_11 | 1 bit | 【Alarm · cell low temperature] Cell temperature is too low, 0: no (or removed) corresponding event, 1: occurred and not lifted |
| Bit_12 | 1 bit | 【Alarm · BMS high temperature】 cell temperature is too low, 0: no (or removed) corresponding event, 1: occurred and not lifted |
| Bit_13 | 1 bit | Keep the field |
| Bit_14 | 1 bit | 【Switch · No charge】 Indicates the allowed / prohibited status of the discharge function 0: Not prohibited (rechargeable) 1: prohibited |
| Bit_15 | 1 bit | 【Switch · prohibited discharge】 indicates the allowable / prohibited status of the discharge function 0: not prohibited (may discharge) 1: prohibited |
| Bit_16 | 1 bit | 【Alarm · Discharge MOS failed】, 0: not failed (or lifted) 1: failed |
| Bit_17 | 1 bit | 【Status · Charging MOS】, 0: off 1: close |
| Bit_18 | 1 bit | 【Status · Discharge MOS】, 0: Off 1: close |
| Bit_19 | 1 bit | 【Alarm · Charge MOS failure】, 0: not failed (or lifted) 1: failure |
| Bit_20 | 1 bit | 【Alarm · High and low voltage cell failure】, 0: no failure (or removed) 1: no failure |
| Bit_21 | 1 bit | 【Alarm · Ultra high temperature failure】, 0: invalid (or removed) 1: invalid |
| Bit_22 | 1 bit | 【Alarm · Large cell pressure difference】, 0: not invalid (or lifted) 1: Large pressure difference |
| Bit_23 | 1 bit | 【Alarm · Battery temperature difference】, 0: not invalid (or lifted) 1: Large temperature difference |

## 4.6  Position Info List

The current GPS location data of the battery reports the latest 10 data

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | explain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length |  |  |  |  |  |  |  | octet 1， |
| Value |  |  |  |  |  |  |  | octet 2~N, each "position" data occupies 21 bytes |

The Value is defined as follows

| Field name | length | explain |
| --- | --- | --- |
| Position Information 1 | 22 Byte | The first position point, see the 4.7 |
| Position Information 2 | 22 Byte | The first position point, see the 4.7 |
| ··· | ··· | The N th position point, see 4.7 |

## 4.7  Position Information

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | explain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length |  |  |  |  |  |  |  | octet 1, Number of data bytes of the value |
| Value |  |  |  |  |  |  |  | octet 2~22, for 21 bytes |

The Value is defined as follows:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | explain |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D1 |  |  |  | D0 |  |  |  | Latitude: Format DD. dddddd Unit: degree Encoding: BCD code |  |
| d1 |  |  |  | d0 |  |  |  |  |  |
| d3 |  |  |  | d2 |  |  |  |  |  |
| d5 |  |  |  | d4 |  |  |  |  |  |
| D1 |  |  |  | D0 |  |  |  | Longitude: format " DDD. dddddd Unit: degree Encoding: BCD code |  |
| d0 |  |  |  | D2 |  |  |  |  |  |
| d2 |  |  |  | d1 |  |  |  |  |  |
| d4 |  |  |  | d3 |  |  |  |  |  |
|  |  | W | S | d5 |  |  |  | W: 1-west longitude 0-E longitude | S: 1-south latitude 0-North Latitude |
| d1 |  |  |  | d0 |  |  |  | Locate date: format "ddmmyy" |  |
| m1 |  |  |  | m0 |  |  |  |  |  |
| y1 |  |  |  | y0 |  |  |  |  |  |
| h1 |  |  |  | h0 |  |  |  | Locate time: format "hhmmss" |  |
| m1 |  |  |  | m0 |  |  |  |  |  |
| s1 |  |  |  | s0 |  |  |  |  |  |
| BD Sat Quality |  |  |  | GPS Sat Quality |  |  |  | Number of Beidou satellites | Number of GPS satellites |
| K1 |  |  |  | K0 |  |  |  | Move speed, format "KKKK.kk" Unit: km / h |  |
| K3 |  |  |  | K2 |  |  |  |  |  |
| k1 |  |  |  | k0 |  |  |  |  |  |
| D1 |  |  |  | D0 |  |  |  | Move direction, "DDDD.d" Unit: degree |  |
| d0 |  |  |  | D2 |  |  |  |  |  |

## 4.8  Cell Voltage List

The current voltage of each cell, each voltage value occupies 2 Bytes, unit mV.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | explain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L ength |  |  |  |  |  |  |  | octet 1, Voltage list data length (bytes) |
| Cell 1 voltage |  |  |  |  |  |  |  | octet 2 ~ 3 |
| Cell 2 voltage |  |  |  |  |  |  |  | octet 4 ~ 5 |
| ··· |  |  |  |  |  |  |  |  |
| Battery N voltage |  |  |  |  |  |  |  | O ctet 2 n~2n+1 |

## 4.9  Temperature List

The temperature measurement value of each temperature detection line with 1 byte of each temperature value address in degrees Celsius ℃?.

Take the maximum (127) / minimum (-128) above the upper and lower limits

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | explain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length |  |  |  |  |  |  |  | octet 1, Data length of cell temperature (number of detection lines * 1) |
| Detection Line 1 temperature value |  |  |  |  |  |  |  | octet 2 |
| Detection Line 2 temperature value |  |  |  |  |  |  |  | octet 3 |
| ···· |  |  |  |  |  |  |  | ···· |
| Detection line N temperature value |  |  |  |  |  |  |  | octet n+1 |

## 4.10 Network Status

Network status of the GPS · 4G module

| Field name | length | explain |
| --- | --- | --- |
| Rssi | 1 Byte | Wireless signal strength (-dbm). If the value of the Rssi field is 140, then the signal strength is -140dbm. 0x00 and 0xFF are both invalid values. |
| PLMN | 6 Bytes | PLMN strings of mobile network base stations, such as "46001F" |
| LAC | 2 Byte s | Location area code |
| CI | 4 Bytes | Cell code (mobile base station code). |
| RAT | 1 Byte | Network system.1:2G network, 8:4G network |

## 4.10 Ext  Lte-N et Status

continue to have

## 4.11  Battery Prop- Ext Day Report Time

Regular reporting time, format hhmmss, if the binary value is "11111111 11111111 11111111" to close the regular reporting function.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | explain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| h h |  |  |  |  |  |  |  | octet 1 ~ 3, regular reporting time: format "hhmmss", 24-hour value. |
| mm |  |  |  |  |  |  |  |  |
| ss |  |  |  |  |  |  |  |  |

## 4.12 Variable- Size Data Format

Variable-long data format

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | explain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length |  |  |  |  |  |  |  | octet 1 |
| Value |  |  |  |  |  |  |  | octet 2~ N |

Length Is the number of bytes of the Value

## 4.13 Date-Time Data Format

Time data format

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | explain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| d1 |  |  |  | d0 |  |  |  | Date: Format "ddmmyy" |
| m1 |  |  |  | m0 |  |  |  |  |
| y1 |  |  |  | y0 |  |  |  |  |
| h1 |  |  |  | h0 |  |  |  | Time: Format "hhmmss" |
| m1 |  |  |  | m0 |  |  |  |  |
| s1 |  |  |  | s0 |  |  |  |  |